🟩 FINAL WIRING CONFIRMATION (USE THIS EXACTLY)

🔷 1. ESP32 ↔ ESP32-CAM (UART1)

| ESP32 Pin | ESP32-CAM Pin |
| --- | --- |
| GPIO 16 (RX1) | TX of CAM |
| GPIO 17 (TX1) | RX of CAM |
| 5V | 5V |
| GND | GND |

✔ CAM → UART1
✔ 115200 baud
✔ ML output will arrive here

🔶 2. DFPlayer MP3-TF-16P (UART2)

| DFPlayer Pin | ESP32 Pin |
| --- | --- |
| VCC | 5V (stable, must be strong 5V) |
| GND | GND |
| TX | GPIO 26 |
| RX | GPIO 27 through 1 kΩ resistor |
| SPK+ / SPK- | Speaker |

✔ DFPlayer → UART2
✔ Works at 9600 baud
✔ Audio only plays if folder structure correct

🔷 3. GPS NEO-6M (SoftwareSerial)

| GPS Pin | ESP32 Pin |
| --- | --- |
| TX | GPIO 15 (GPS → ESP32 RX) |
| RX | GPIO 4 (ESP32 TX → GPS) |
| VCC | 5V or 3.3V |
| GND | GND |

✔ GPS uses SoftwareSerial (15,4)
✔ Baud 9600

🔶 4. GSM SIM800L (SoftwareSerial)

| GSM Pin | ESP32 Pin |
| --- | --- |
| TX | GPIO 32 |
| RX | GPIO 33 |
| VCC | 3.7V–4.2V (battery or buck converter — NOT 5V) |
| GND | GND |

⚠ VERY IMPORTANT:
SIM800L cannot run from 5V
You MUST give:

- 3.7V Li-ion battery OR

- Buck converter (12V → 4V) set to 4.0V

🔷 5. MAX30102 (I2C)

| MAX30102 Pin | ESP32 Pin |
| --- | --- |
| VIN | 3.3V |
| GND | GND |
| SDA | GPIO 21 |
| SCL | GPIO 22 |

✔ MAX30102 → 3.3V ONLY
✔ Reads IR & HR correctly

🔶 6. MPU6050 (I2C Shared Bus)

| MPU6050 Pin | ESP32 Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| SDA | GPIO 21 |
| SCL | GPIO 22 |

✔ Shares I2C with MAX30102
✔ Works perfectly

🔷 7. Ultrasonic HC-SR04

| HC-SR04 | ESP32 |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TRIG | GPIO 12 |
| ECHO | GPIO 14 → Voltage Divider Required |

⚠ ESP32 ECHO must be 3.3V MAX

Use:

- 10kΩ + 15kΩ divider